Material Safety Data Sheet

acc. to OSHA and ANSI

| Printing date 05/30/2000 | Reviewed on 05/05/2000 |
| --- | --- |

1 Identification of substance: - Product details: - Trade name: Hydrogen peroxide, 29.0-32.0% - Stock number: 33323 - Manufacturer/Supplier: Alfa Aesar, A Johnson Matthey Company Johnson Matthey Catalog Company, Inc. 30 Bond Street Ward Hill, MA 01835-8099 Emergency Phone: (978) 521-6300 CHEMTREC: (800) 424-9300 Web Site: www.alfa.com - Information department: Health, Safety and Environmental Department - Emergency information: During normal hours the Health, Safety and Environmental Department. After normal hours call Chemtrec at (800) 424-9300.

2 Composition/Data on components: - Chemical characterization: Description: (CAS#) Hydrogen peroxide (CAS# 7722-84-1), 29.0-32.0% Water (CAS# 7732-18-5), balance - Identification number(s): - EINECS Number: 231-765-0 - EU Number: 008-003-00-9

3 Hazards identification - Hazard description: C Corrosive O Oxidizing - Information pertaining to particular dangers for man and environment R 8 Contact with combustible material may cause fire. R 34 Causes burns.

4 First aid measures - General information Immediately remove any clothing soiled by the product. - After inhalation Supply fresh air. If required, provide artificial respiration. Keep patient warm. Seek immediate medical advice. - After skin contact Immediately wash with water and soap and rinse thoroughly. Seek immediate medical advice. - After eye contact Rinse opened eye for several minutes under running water. Then consult a doctor. - After swallowing Seek immediate medical advice.

5 Fire fighting measures - Suitable extinguishing agents Water spray - Special hazards caused by the material, its products of combustion or resulting gases: Promotes fire Danger of containers bursting upon heating. - Protective equipment: Wear self-contained respirator. Wear fully protective impervious suit.

6 Accidental release measures - Person-related safety precautions: Wear protective equipment. Keep unprotected persons away. Ensure adequate ventilation - Measures for environmental protection: Do not allow material to be released to the environment without proper governmental permits. - Measures for cleaning/collecting: Absorb with liquid-binding material. Use neutralizing agent. Dispose contaminated material as waste according to item 13. Ensure adequate ventilation. - Additional information: See Section 7 for information on safe handling See Section 8 for information on personal protection equipment. See Section 13 for disposal information.

7 Handling and storage - Handling - Information for safe handling: Keep container tightly sealed. Store in cool, dry place in tightly closed containers. Ensure good ventilation at the workplace. - Information about protection against explosions and fires: Substance/product can reduce the ignition temperature of flammable substances. - Storage - Requirements to be met by storerooms and receptacles: No special requirements. - Information about storage in one common storage facility: Store away from flammable substances. Store away from reducing agents. Store away from metals. - Further information about storage conditions: Keep container tightly sealed. Store in cool, dry conditions in well sealed containers. Protect from exposure to light. Store under lock and key and with access restricted to technical experts or their assistants only.

8 Exposure controls and personal protection - Additional information about design of technical systems: Properly operating chemical fume hood designed for hazardous chemicals and having an average face velocity of at least 100 feet per minute. Components with limit values that require monitoring at the workplace: Hydrogen peroxide ppm ACGIH TLV 1 Belgium TWA 1 Denmark TWA 1 Finland TWA 1; 3-STEL France TWA 1 Germany TWA 1 Ireland TWA 1; 2-STEL Netherlands TWA 1 Switzerland TWA 1; 2-STEL United Kingdom TWA 1; 2-STEL OSHA PEL 1 - Additional information: No data - Personal protective equipment - General protective and hygienic measures The usual precautionary measures for handling chemicals should be followed. Keep away from foodstuffs, beverages and feed. Remove all soiled and contaminated clothing immediately. Wash hands before breaks and at the end of work. Avoid contact with the eyes and skin. - Breathing equipment: Use suitable respirator when high concentrations are present. - Protection of hands: Impervious gloves - Eye protection: Safety glasses Tightly sealed goggles Full face protection - Body protection: Protective work clothing.

9 Physical and chemical properties: - Form: Liquid - Color: Colorless - Odor: Pungent - Value/Range Unit Method - Change in condition - Melting point/Melting range: -41 ° C - Boiling point/Boiling range: 150.2 ° C - Sublimation temperature / start: Not determined - Flash point: Not applicable - Flammability (solid, gaseous) Contact with combustible material may cause fire. - Ignition temperature: Not determined - Decomposition temperature: Not determined - Explosion limits: - Lower: Not determined - Upper: Not determined - Vapor pressure: at 20 ° C 30.99 hPa - Density: at 20 ° C 1.45 g/cm3 - Solubility in / Miscibility with - Water: Fully miscible

10 Stability and reactivity - Thermal decomposition / conditions to be avoided: Decomposition will not occur if used and stored according to specifications. - Materials to be avoided: Reducing agents, easily oxidized materials Metal powders Light - Dangerous reactions Reacts with reducing agents Reacts with flammable substances - Dangerous products of decomposition: Oxygen Heat

11 Toxicological information - Acute toxicity: LD/Lc50 values that are relevant for classification: Oral: LDLo: 1429 mg/kg (man) LDLo: 2626 ug/kg (wmn) - Primary irritant effect: - on the skin: Corrosive effect on skin and mucous membranes. Irritant to skin and mucous membranes. - on the eye: Strong corrosive effect. Irritating effect. - Sensitization: No sensitizing effects known. - Subacute to chronic toxicity: Hydrogen peroxide is highly irritating to skin, eyes and mucous membranes. The eyes in particular are especially sensitive. Irritation is a function of concentration and solutions over 30% may easily cause skin blisters. - Additional toxicological information: Swallowing will lead to a strong corrosive effect on mouth and throat and to the danger of perforation of esophagus and stomach. To the best of our knowledge the acute and chronic toxicity of this substance is not fully known. IARC-3: Not classifiable as to carcinogenicity to humans. ACGIH A3: Animal carcinogen: Agent is carcinogenic in experimental animals at a relatively high dose, by route(s) of administration, at site(s), of histologic type(s), or by mechanism(s) not considered relevant to worker exposure. Available epidemologic studies do not confirm an increased risk of cancer in exposed humans. Available evidence suggests that the agent is not likely to cause cancer in humans except under uncommon or unlikely routes or levels of exposure.

12 Ecological information: - General notes: Do not allow material to be released to the environment without proper governmental permits.

13 Disposal considerations - Product: - Recommendation Consult state, local or national regulations for proper disposal. - Uncleaned packagings: - Recommendation: Disposal must be made according to official regulations.

14 Transport information - DOT regulations: - Hazard class: 5.1 - Identification number: UN2014 - Packing group: II - Proper shipping name (technical name): Hydrogen peroxide, aqueous solution - Land transport ADR/RID (cross-border) - ADR/RID class: 5.1 Oxidizing substances - Item: 1b - Danger code (Kemler): 58 - UN-Number: 2014 - Description of goods: Hydrogen peroxide, aqueous solution - Maritime transport IMDG: - IMDG Class: 5.1 - UN Number: 2014 - Packaging group: II - Proper shipping name: Hydrogen peroxide, aqueous solution - Air transport ICAO-TI and IATA-DGR: - ICAO/IATA Class: 5.1 - UN/ID Number: 2014 - Packaging group: II - Proper shipping name: Hydrogen peroxide, aqueous solution

15 Regulations - Product related hazard informations: - Hazard symbols: C Corrosive O Oxidizing - Risk phrases: 8 Contact with combustible material may cause fire. 34 Causes burns. - Safety phrases: 3 Keep in a cool place. 28 After contact with skin, wash immediately with plenty of water 36/39 Wear suitable protective clothing and eye/face protection. 45 In case of accident or if you feel unwell, seek medical advice immediately. - National regulations All components of this product are listed in the U.S. Environmental Protection Agency Toxic Substances Control Act Chemical Substance Inventory. - Information about limitation of use: For use only by technically qualified individuals.

16 Other information: Employers should use this information only as a supplement to other information gathered by them, and should make independent judgement of suitability of this information to ensure proper use and protect the health and safety of employees. This information is furnished without warranty, and any use of the product not in conformance with this Material Safety Data Sheet, or in combination with any other product or process, is the responsibility of the user. - Department issuing MSDS: Health, Safety and Environmental Department. - Contact: Darrell R. Sanders